MCXE247 Clock Configuration

Technical Reference: Initialization, Safety Diagnostics & Debugging

IEC 60730 Clock Safety Validation

# 1  System Overview

## 1.1  Purpose

This document describes the manual clock bring-up, register configuration, clock tree analysis, IEC 60730 safety diagnostics, and debugging procedure for the NXP MCXE247 microcontroller.

The target application domain is railway CBTC (Communications-Based Train Control) systems, which impose the following constraints on the clock subsystem:

- Deterministic — clock source and frequency must be fixed at startup.

- Stable — no runtime clock switching or dynamic frequency scaling.

- Monitored — clock output is measured against reference.

- Fault-detectable — any clock deviation must be caught.

## 1.2  Clock Philosophy

The implementation avoids SDK drivers and configures registers directly. This ensures:

- Explicit register-level traceability for functional safety analysis.

- Fail-stop behaviour — clock faults produce a deterministic system halt, not a fallback.

- No hidden runtime reconfiguration by middleware.

| Feature | NXP SDK | Custom (This Impl.) | Notes |
| --- | --- | --- | --- |
| Uses SDK drivers | Yes | No | Direct register access |
| Deterministic | No | Yes | Fixed sequence |
| Fail-stop | No | Yes | No silent fallback |
| Runtime reconfiguration | Yes | No | Not permitted |
| Suitable for SIL-4 | ❌ No | ⚠ Possible (with improvements) | Needs diagnostic coverage analysis |
| Clock fallback allowed | Yes | No | Fail-stop required |
| Explicit register access | No | Yes | Full traceability |
| Timing traceability | Hard | Clear | Each step documented |
| Safety philosophy | General-purpose | Deterministic |  |

Table 1 — NXP SDK vs. Custom Implementation Comparison

# 2  Hardware Architecture

## 2.1  System Clock Generator (SCG)

The SCG is the central clock management block on the MCXE247. It integrates:

- FIRC — Fast Internal RC oscillator (~48 MHz). Used as safe startup clock.

- SIRC — Slow Internal RC oscillator (~8 MHz).

- SOSC — System Oscillator. Accepts external crystal or TCXO input.

- SPLL — System Phase-Locked Loop. Multiplies SOSC output to generate high-speed core clock.

## 2.2  SOSC (System Oscillator)

The SOSC block accepts an external clock source. Two modes are supported:

- Crystal mode — MCXE247 drives the crystal directly.

- Bypass mode — external TCXO signal drives EXTAL pin directly. Used in this implementation for deterministic frequency accuracy.

## 2.3  SPLL (System PLL)

The SPLL multiplies the SOSC output to produce the high-speed system clock:

- Input: SOSC output (post-PREDIV divider).

- VCO: Input × MULT → internal high-frequency signal.

- Output: VCO ÷ 2 (fixed post-divider).

## 2.4  Clock Tree

The peripheral clocks (SPLLDIV1, SPLLDIV2) are derived from SPLL output via independent dividers. Peripheral modules are gated separately through PCC (Peripheral Clock Controller) and SIM.

Figure 1 — MCXE247 Clock Tree (Top Level)

Figure 2 — MCXE247 Clock Tree (Detailed)

Figure 3 — MCXE247 System Overview Diagram

Figure 4 — SCG Block Diagram

| Concept | Meaning |
| --- | --- |
| SIM | Enables some system-level clocks |
| PCC | Enables peripheral clocks |
| CGC bit | Turns ON peripheral clock gate |
| PR bit | Peripheral ready indicator |
| CLOCK_EnableClock() | Generic SDK call to enable any peripheral clock |

Table 2 — Clock Control Concept Reference

# 3  Clock Initialization Sequence

The Clock_Init() function follows a strict bring-up order to avoid operating any peripheral at an unsafe frequency during reconfiguration.

Figure 5 — Clock_Init() Flowchart

Figure 6 — Clock Initialization Step Detail

## 3.1  Step-by-Step Sequence

- Start from safe internal clock (FIRC)

SCG->FIRCCSR |= ... FIRCEN

Wait FIRCVLD = 1

Ensures MCU has a stable internal clock before touching any other source.

- Switch system clock to FIRC

SCG->RCCR = SCS(3) ...

Wait CSR[SCS] == 3

Forces system clock to FIRC so that subsequent oscillator and PLL changes do not affect the running core.

- Configure and enable SOSC

Disable SOSC

Set SOSCCFG (bypass mode, range, etc.)

Enable SOSC

Wait SOSCVLD = 1

Configures external TCXO path in bypass mode and waits until hardware reports stable oscillator output.

- Set flash clock divider

FTFC->FCLKDIVR = ...

Sets flash clock divider early so flash does not exceed its maximum operating frequency after PLL switch.

- Configure, enable, and lock SPLL

Disable SPLL

Set SPLLCFG (PREDIV, MULT)

Set SPLLDIV (peripheral clock dividers)

Enable SPLL

Wait SPLLVLD = 1

Programs PLL from SOSC input and waits for lock confirmation.

- Switch system clock to SPLL

SCG->RCCR = SCS(6) with core/bus/slow dividers

Wait CSR[SCS] == 6

Switches system clock source to SPLL and configures final core, bus, and flash dividers atomically.

- Update SystemCoreClock

SystemCoreClock = CLOCK_CORE_HZ

Updates the software variable to reflect actual CPU frequency.

# 4  Register Configuration

## 4.1  SOSCCFG

Configures SOSC oscillator mode and frequency range. Key fields:

- EREFS — External reference select: 1 = bypass (external TCXO on EXTAL).

- RANGE — Frequency range select: must match crystal/TCXO frequency band.

## 4.2  SOSCCSR

SOSC Control and Status Register. Key fields:

- SOSCEN (bit 0) — Enable SOSC.

- SOSCVLD (bit 24) — SOSC output valid (set by hardware when stable).

- SOSCSEL (bit 16) — SOSC selected as clock source.

Verified value: 0x01000001 (enabled + valid).

## 4.3  SPLLCFG

PLL multiplication and pre-division configuration. Key fields:

- PREDIV[2:0] — Pre-divider. Divides SOSC input before VCO.

- MULT[4:0] — VCO multiplication factor.

For EXTAL = 8 MHz: PREDIV = 0 (÷1), MULT = 31 (×32) → VCO = 256 MHz → output = 128 MHz.

For EXTAL = 16 MHz: PREDIV = 1 (÷2), MULT = 31 (×32) → VCO = 256 MHz → output = 128 MHz.

Verified value: 0x00100000

## 4.4  SPLLCSR

PLL Control and Status Register. Key fields:

- SPLLEN (bit 0) — Enable SPLL.

- SPLLVLD (bit 24) — PLL locked and output valid.

- SPLLSEL (bit 16) — SPLL selected as system clock.

Verified value: 0x03000001 (enabled + locked + selected).

## 4.5  RCCR

Run Clock Control Register. Selects system clock source and sets core/bus/flash dividers. Key fields:

- SCS[3:0] — System Clock Source: 3 = FIRC, 6 = SPLL.

- DIVCORE[3:0] — Core clock divider (value = divider − 1).

- DIVBUS[3:0] — Bus clock divider.

- DIVSLOW[3:0] — Slow/flash clock divider.

Verified value: 0x06010013

## 4.6  SPLLDIV

SPLL Output Clock Dividers for peripheral clock buses. Fields:

- SPLLDIV1[2:0] — Divider for SPLLDIV1 peripheral clock output.

- SPLLDIV2[2:0] — Divider for SPLLDIV2 peripheral clock output.

Verified value: 0x00000000 (dividers set to ÷1 by default).

Figure 7 — Register Configuration Screenshot

# 5  Clock Tree Analysis

## 5.1  EXTAL = 8 MHz

| Clock Domain | Calculation | Result |
| --- | --- | --- |
| SOSC | Crystal input | 8 MHz |
| PLL input | 8 MHz (PREDIV ÷1) | 8 MHz |
| PLL VCO | 8 × 32 (MULT = 31+1) | 256 MHz |
| PLL output | VCO ÷ 2 (fixed) | 128 MHz |
| Core clock | 128 ÷ 2 (DIVCORE) | 64 MHz |
| Bus clock | 128 ÷ 4 (DIVBUS) | 32 MHz |
| Flash clock | 128 ÷ 6 (DIVSLOW) | 21.33 MHz |
| SPLLDIV1 | 128 ÷ 4 | 32 MHz |
| SPLLDIV2 | 128 ÷ 4 | 32 MHz |

Table 3 — Clock Frequencies for EXTAL = 8 MHz

## 5.2  EXTAL = 16 MHz (TCXO with PREDIV ÷2)

| Clock Domain | Calculation | Result |
| --- | --- | --- |
| SOSC | TCXO input | 16 MHz |
| PLL input | 16 ÷ 2 (PREDIV) | 8 MHz |
| PLL VCO | 8 × 32 (MULT = 31+1) | 256 MHz |
| PLL output | VCO ÷ 2 (fixed) | 128 MHz |
| Core clock | 128 ÷ 2 (DIVCORE) | 64 MHz |
| Bus clock | 128 ÷ 4 (DIVBUS) | 32 MHz |
| Flash clock | 128 ÷ 6 (DIVSLOW) | 21.33 MHz |
| SPLLDIV1 | 128 ÷ 4 | 32 MHz |
| SPLLDIV2 | 128 ÷ 4 | 32 MHz |

Table 4 — Clock Frequencies for EXTAL = 16 MHz

Both configurations produce identical final frequencies. The PREDIV field compensates for a higher-frequency TCXO.

# 6  Safety Diagnostics

## 6.1  IEC 60730 Clock Test

The IEC 60730 clock test is a runtime diagnostic that verifies the system clock frequency is within a defined tolerance band. It does not reconfigure clocks — it verifies the result of the initialization sequence.

## 6.2  What the Test Detects

### 6.2.1  PLL Runaway

If the PLL MULT register is corrupted or misbehaves, the measured frequency will fall outside the MIN/MAX window.

### 6.2.2  Crystal Drift

If the external 8 MHz crystal deviates beyond its tolerance band, the test detects the deviation.

### 6.2.3  Divider Corruption

If RCCR divider fields are modified unexpectedly after initialization, the core clock will shift and the test will fail.

### 6.2.4  Clock Switching Faults

If SCS field was not correctly switched (e.g., system remained on FIRC instead of SPLL), frequency will not match expected.

### 6.2.5  Periodic Runtime Supervision

The test can be re-run at periodic intervals (e.g., during background task) to increase diagnostic coverage over mission time.

## 6.3  Frequency Window Validation Mechanism

The test counts system clock ticks against an independent reference clock window, then checks:

MIN_FREQ < measured_freq < MAX_FREQ

If the measured frequency is outside this range, the test reports FAIL. The reference clock is typically FIRC, LPO, or an independent hardware timer — a source independent of SOSC/SPLL.

## 6.4  What the Test Does NOT Detect

- Incorrect initialization sequence (e.g., wrong wait order).

- Missing VALID bit waits during initialization.

- Incorrect flash clock divider (DIVSLOW).

- Register access order violations.

The test validates frequency output behaviour only, not register structural correctness.

## 6.5  Open Safety Questions (Unresolved)

- Is reference clock truly independent of SOSC/SPLL?

- What is the tolerance justification for MIN/MAX window?

- What action is taken on failure — safe state or interrupt?

- Is the measurement timeout bounded?

- Has diagnostic coverage been formally analyzed?

Figure 8 — IEC 60730 Clock Test Block Diagram

Figure 9 — IEC 60730 Frequency Window Validation

# 7  Debugging Procedure

## 7.1  Register Debugging Flow

When a clock problem occurs (system stuck, wrong frequency, PLL not locking), check these six registers in the following order. If all six are correct, the clock system is functioning correctly.

| Step | Register | Check | Failure Meaning |
| --- | --- | --- | --- |
| 1 | SCG->CSR | SCS == expected value | Clock switch failed — system on wrong source |
| 2 | SCG->SOSCCSR | SOSCVLD == 1 | Crystal/TCXO not starting or unstable |
| 3 | SCG->SPLLCSR | SPLLVLD == 1 | PLL not locking — check SOSCCFG/SPLLCFG |
| 4 | SCG->SPLLCFG | PREDIV, MULT match expected | Wrong multiplication or pre-division factor |
| 5 | SCG->RCCR | SCS, DIVCORE, DIVBUS, DIVSLOW correct | Wrong divider — frequency mismatch on core/bus |
| 6 | SCG->SPLLDIV | SPLLDIV1, SPLLDIV2 correct | Peripheral clock frequency wrong |

Table 5 — 6-Step Clock Debugging Register Sequence

## 7.2  Clock Failure Diagnosis Steps

### Step 1 — Check System Clock Source

Read: SCG->CSR

Extract SCS field. If SCS ≠ 6 (SPLL not selected), the clock switch in RCCR did not complete. Check that the RCCR write occurred after SPLLVLD = 1.

### Step 2 — Check Oscillator Valid

Read: SCG->SOSCCSR

If SOSCVLD = 0, the external oscillator has not started. Check hardware: crystal connections, EREFS mode, RANGE setting, TCXO drive level.

### Step 3 — Check PLL Lock

Read: SCG->SPLLCSR

If SPLLVLD = 0, PLL has not locked. Verify SOSC is valid first. Then check SPLLCFG PREDIV and MULT values for correctness.

### Step 4 — Check PLL Configuration

Read: SCG->SPLLCFG

Verify PREDIV and MULT fields match the expected frequency calculation. Incorrect MULT causes PLL VCO to target wrong frequency.

### Step 5 — Check System Dividers

Read: SCG->RCCR

Verify DIVCORE, DIVBUS, DIVSLOW match the intended divider values. Wrong dividers cause core or peripheral frequency errors without changing the clock source.

### Step 6 — Check Peripheral Clock Dividers

Read: SCG->SPLLDIV

Verify SPLLDIV1 and SPLLDIV2 match expected peripheral clock dividers.

Figure 10 — GDB Register Read — SOSCCSR / SPLLCSR

Figure 11 — GDB Register Read — SPLLCFG / SPLLDIV

# 8  Verification Results

## 8.1  Register Comparison Table

The following table compares register values as expected by the clock configuration tool against values read back via GDB at runtime.

| Register | Config Tool Value | GDB (Actual) | Match? |
| --- | --- | --- | --- |
| SPLLCFG | 0x00100000 | 0x00100000 | ✅ |
| SPLLDIV | 0x00000000 | 0x00000000 | ✅ |
| RCCR | 0x06010013 | 0x06010013 | ✅ |
| CSR | 0x06010013 | 0x06010013 | ✅ |
| SPLLCSR | Locked | 0x03000001 | ✅ |
| SOSCCSR | Valid | 0x01000001 | ✅ |

Table 6 — Register Verification: Config Tool vs. GDB Readback

## 8.2  Debug Output Validation

Serial debug output captured after Clock_Init() completion:

===== MCX Clock Tree =====

SystemCoreClock: 64000000 Hz

System Clock Source: 6

SOSC CSR: 0x1030001

SPLL CSR: 0x3030001

SPLL CFG: 0x100000

SPLL DIV: 0x202

Core Divider: 1

Bus Divider: 1

Flash Divider: 2

==========================

All values match the clock tree analysis in Section 5. The configuration is confirmed correct for EXTAL = 8 MHz.

Figure 12 — Clock Register Verification Debug Output

## 8.3  Confirmation of Diagram Match

The register values and frequency calculations in Sections 4 and 5 exactly match the clock tree diagram (Figure 1 / Figure 2). The initialization sequence produces the expected clock topology without deviation.